How to Change a Flat Tire

When you get a flat tire and you don’t know how to change it quickly and safely, you’re stuck.  Do you really want to call a tow truck for a blowout?

- If you are driving, pull off the road to a safe place where there is good visibility for other drivers.  Make SURE the ground is level – never attempt to change a tire on an incline.

- Put on your hazards, put the car into PARK, and exit all passengers.

- Remove the spare tire, tire iron, and jack from the trunk.

- If the wheel has a hubcap, remove it with a screwdriver or the beveled end of the tire iron.

- Use the lug-wrench end of the tire iron to loosen two pairs of nuts diagonally opposite each other.

- Place the jack under the car behind the wheel.  Check your manual to see where it should go, otherwise you could damage the trim.

- Jack up the car until the wheel is about SIX INCHES off the ground.

- Remove the nuts and slide the wheel off. Put the nuts in your pocket or in the hubcap to stop them from rolling away (or worse, rolling beneath the car).

- Line up the holes in the center of the spare tire rim with the bolts, and slide on the tire as far as it will go. Hand-tighten the nuts.

- Lower the car and remove the jack, THEN tighten the nuts completely using the tire iron, working with diagonal opposite pairs, as before.

- Replace the hubcap and secure the flat tire and tools in the trunk. Visit a garage ASAP.